# Secure Networking in VANETs

# Introduction

VANETs use multiple radio access technologies (RATs) existing On-Board Units (OBUs), such as ITS-G5, Wi-Fi, cellular networks. Multihop communications extend the connectivity range of OBUs, relying on the software defined networks (SDN) paradigm for an efficient resource management and to tackle the challenges of the dynamicity in VANETs (mobility, distribution of resources). Resiliency in the communications can be pursued through multihop and multipath approaches and protocols that consider the network conditions and service requirements to, in real-time, decide the best multipath between vehicles and to the infrastructure, directly or through a chain of vehicles.

The paradigm of Vehicle to everything (V2X), where vehicles communicate with other devices/objects/things can introduce safety and security concerns. If information is not provided in real-time than the collaborative situational awareness will provide an incorrect perception of risk and will not provide insightful insights to drivers to be able to avoid accidents. On the other hand, the security issues can include attacks with false information, where malicious vehicles use false identities to send information, message suspension attacks, where vehicles drop messages sent by other vehicles having an impact in safe driving and in cooperation environments. In this regard the standard evolution like the 802.11bh and 802.11bi introduce support for Medium Access Control (MAC) address randomization and to shed light in the privacy issues that exist in using static MAC addresses, as used in current solutions. Tracking vehicles/objects is facilitated when using static identifiers, which is not compliant with privacy regulations.

This project will research secure networking aspects in VANETs, which will focus on the secure transmission of messages, on the efficient and scalable management of cryptographic material to secure communication protocols in VANETs, among other aspects.

The project aims to be conceived in two main phases:

- Enable the communication of vehicles using a communication stack with different protocols like CAM, DENM, VAM, among others.

- Enable secure communication using the protocol stack assessed/configured in the previous phase.

# Objectives

The major goal of the project is the implementation of security in communication protocols of VANETs.

The goals of this project are:

- Deployment of communication protocols in a simple testbed .

- Evaluation of communication protocols in a simple testbed.

- Deployment of security solutions for communication protocols relying on TLS, Noise protocols.

- Impact Assessment of security solutions in VANETs communication protocols

# Plan

The plan is depicted in the table below.

| Task | Description / Goal | Duration | Expected Output |
| --- | --- | --- | --- |
| T1 – Testbed preparation | Prepare the testbed to deploy Communication protocols | 2 Weeks | - Testbed - Configuration files |
| T2 – Communication Protocols | Deploy Communication protocols on testbed, using different protocols and messages | 3 weeks | - Configuration files - ‘Scripts’ to send messages |
| T3 – Evaluation | Evaluate deployed communication protocols | 2 weeks | - Benchmarking methodology - ‘Scripts’ to run benchmark - Performance Baseline |
| T4 – Secure Communication Protocols - TLS | Deploy secure communication protocols relying on TLS solutions | 3 weeks | - Configuration files - ‘Scripts’ to send messages |
| T5 – Evaluation of Secure Protocols | Evaluate deployed secure communication protocols | 2 weeks | - Benchmarking methodology - ‘Scripts’ to run benchmark - Comparison with Performance Baseline |
| T6 – Secure Communication Protocols – Noise Protocol | Deploy secure communication protocols relying on Noise Protocol solutions | 3 weeks | - Configuration files - ‘Scripts’ to send messages |
| T7 – Evaluation of Secure Protocols - Noise | Evaluate deployed secure communication protocols | 2 weeks | - Benchmarking methodology - ‘Scripts’ to run benchmark - Comparison with previous evaluations & Baseline |
| T8 – Write Scientific Paper | Write a scientific paper | 2 weeks | - Scientific Paper - Github Repository - (Possible contributions to Vanetza-NAP project) |

# References

- Vanetza-NAP, available at: https://github.com/nap-it/vanetza-nap?tab=readme-ov-file